# Wiring Diagram & Comprehensive Explanation

# 1. Overview

The system relies on a Raspberry Pi 4B connected to a USB Camera, AMG8833 Thermal Sensor, DHT22 Sensor, and GSM900A module.

# 2. AMG8833 Thermal Camera (I2C)

VIN to Pi 3.3V (Pin 1)

GND to Pi GND (Pin 6)

SDA to Pi GPIO2/SDA1 (Pin 3)

SCL to Pi GPIO3/SCL1 (Pin 5)

# 3. DHT22 Ambient Sensor (GPIO)

VCC to Pi 3.3V or 5V

GND to Pi GND

DATA to Pi GPIO4 (Pin 7) (requires 10k pull-up resistor to VCC)

# 4. GSM900A Module (UART)

TX to Pi RXD / GPIO15 (Pin 10)

RX to Pi TXD / GPIO14 (Pin 8)

GND to Pi GND