# Bill of Materials (BOM)

Structural Components · Electronics · Enclosure & Fabrication

This document provides a comprehensive list of all materials used in the construction of the Offline AI Swine Health Monitoring System. The system is divided into two major categories: Structural Components (the physical enclosure and mounting hardware) and Electronic Components (the sensors, compute unit, and power management hardware).

# 1. Structural Components

The structural framework and enclosure are designed for durability in a farm environment. Steel bar stock provides a rigid mounting structure for the electronics enclosure, while a 3D-printed enclosure houses the camera and thermal sensor in a unified, weather-resistant body.

| Item | Material / Spec | Purpose | Notes |
| --- | --- | --- | --- |
| Flat Bar | Mild Steel Flat Bar | Main mounting frame and bracket fabrication for attaching the device to pen railings or posts | Cut and drilled to custom dimensions; welded or bolted at joints |
| Round Bar | Mild Steel Round Bar | Pivot arms, support rods, and adjustable tilt brackets for the camera enclosure | Provides adjustable camera angle for optimal pen coverage |
| Bolts & Nuts | Stainless Steel (M4, M5, M6 assorted) | All mechanical fastening points — frame joints, enclosure mounting, component securing | Stainless steel used to resist corrosion from farm moisture and cleaning agents |
| Washers | Stainless Steel (M4, M5, M6 assorted) | Load distribution under bolt heads to prevent pull-through on plastic and sheet metal | Spring washers used on vibration-prone joints |
| 3D-Printed Enclosure | PETG Filament (combined camera + thermal unit) | Single unified enclosure housing the Logitech 1080p webcam and the AMG8833 thermal camera side-by-side with a shared mounting point | PETG chosen for its high temperature resistance (~80°C HDT), UV stability, and superior layer adhesion vs. PLA — essential for outdoor/farm deployment |

# 2. Electronic Components

All electronic components were selected for their compatibility with the Raspberry Pi ecosystem, availability, and suitability for edge deployment in a livestock monitoring context.

| Component | Model / Spec | Quantity | Role in System |
| --- | --- | --- | --- |
| Single-Board Computer | Raspberry Pi 4B (4 GB RAM) | 1 | Central compute unit. Runs the AI inference pipeline, Flask web server, SORT tracker, risk engine, and all sensor interfaces. 4 GB RAM is sufficient for YOLOv8n ONNX inference without swap. |
| USB Camera | Logitech C920 / 1080p Webcam | 1 | Primary vision sensor. Captures the pig pen at 1080p. OpenCV downscales frames to 640x480 internally for YOLO inference. Mounts inside the 3D-printed enclosure alongside the AMG8833. |
| Thermal Imaging Sensor | Adafruit AMG8833 (8x8 Grid) | 1 | Provides a low-resolution (8x8 pixel) thermal map of the pen. Used to estimate individual pig body temperature zones. Communicates via I2C at address 0x69. Mounted co-located with the USB camera. |
| Ambient Temp/Humidity Sensor | DHT22 (AM2302) | 1 | Measures ambient barn temperature and relative humidity. Used to calculate the Temperature-Humidity Index (THI) and to provide a baseline for fever detection. Connects via single-wire GPIO. |
| GSM Module | GSM900A (SIM800L-compatible) | 1 | Sends SMS health alerts to the farmer when a risk event is detected. Communicates with the Pi via UART at 9600 baud using standard AT commands. Requires a SIM card with SMS capability. |
| Battery Cells | 18650 Lithium-Ion Cells (3S3P configuration) | 6 cells | 6x 18650 cells arranged in a 3S3P configuration: 3 cells in series (11.1 V nominal) x 3 in parallel (~7 Ah total). Provides the primary power storage for off-grid operation. |
| Battery Management System | 3S 20A BMS | 1 | Protects the 3S lithium battery pack from over-charge (>12.6 V), over-discharge (<9.0 V), over-current, and short-circuit conditions. Essential for safe lithium battery operation. |
| Solar Charger | CN3791 MPPT Solar Charger (12 V) | 1 | Maximum Power Point Tracking (MPPT) solar charge controller. Efficiently harvests energy from the 12 V solar panel and charges the 3S battery pack. Extends off-grid runtime significantly. |
| Solar Panel | 12 V Polycrystalline / Monocrystalline Solar Panel | 1 | Primary renewable energy source. 12 V output feeds the CN3791 MPPT charger. Panel wattage (e.g., 10W–20W) should be sized to balance daily energy consumption of the system (~10–15 Wh/day typical). |
| DC-DC Buck Converter | XL4016 Step-Down Buck Converter | 1 | Steps down the 11.1–12.6 V battery voltage to a stable 5.1 V output for powering the Raspberry Pi and all 5V peripherals. The output voltage is adjustable via a trim potentiometer. Must be pre-set to 5.1 V before connecting the Pi. |
| Master Power Switch | Rocker Switch (SPST, rated >= 5A) | 1 | Placed on the positive battery rail between the BMS P+ output and the XL4016 buck converter input. Acts as the main on/off switch for the entire system without disconnecting the solar charger or BMS. |