# Comprehensive Code Explanation

Offline AI Swine Health Monitoring System · Source Code Reference

# 1. Project Structure Overview

The system is written entirely in Python 3.11 and organised into a clean modular monorepo. Each directory has a single responsibility, making the codebase easy to navigate, test, and extend.

All production code lives under src/. Utility scripts (training, evaluation, export) live under scripts/. Tests are in tests/. Hardware never runs on the PC — src/ is designed to run on the Raspberry Pi 4B.

| File / Module | Description |
| --- | --- |
| src/main.py | System entry point. Initialises all subsystems and runs the main processing loop. |
| src/config_loader.py | Loads and validates config/config.yaml at startup. |
| src/inference/detector.py | ONNX Runtime inference wrapper for YOLOv8n. Handles preprocessing, session management, and NMS postprocessing. |
| src/tracking/sort_tracker.py | SORT algorithm implementation: Kalman filter + Hungarian algorithm for detection-to-track assignment. |
| src/tracking/pig_tracker.py | Stateful pig ID manager. Maintains per-track behavior, stationary timer, and centroid history. |
| src/thermal/thermal_reader.py | AMG8833 I2C interface using adafruit_amg88xx. Returns 8×8 numpy float array and provides bilinear upscaled 32×32 grid for heatmap rendering. |
| src/thermal/thermal_mapper.py | Maps YOLO bounding box centroids to AMG8833 grid zones. Returns the zone temperature for each pig ID. |
| src/hardware/async_camera.py | Non-blocking USB camera reader. Runs a background daemon thread that continuously captures frames into a shared buffer, eliminating 43 ms blocking I/O from the main inference loop. |
| src/hardware/dht22_sensor.py | DHT22 ambient sensor reader. Computes the Temperature Humidity Index (THI) from temperature and humidity readings. |
| src/hardware/gsm_notifier.py | GSM900A AT-command driver over UART. Sends SMS alerts with a configurable 5-minute cooldown to prevent alert flooding. |
| src/health/risk_engine.py | Dual-channel hybrid risk evaluation engine. Combines individual (Channel 1) and population-level (Channel 2) anomaly detection with THI-adaptive thresholds. |
| src/analytics/behavior_analyzer.py | Tracks stationary duration per pig ID and computes the herd-level lethargy ratio (proportion of stationary pigs). |
| src/analytics/pig_counter.py | Occupancy-based pig counter. Returns current active pig count without inflating on re-entry (corrects SORT's cumulative ID assignment). |
| src/database/schema.py | SQLite database schema with WAL mode, busy_timeout, and three tables: ambient_readings, pen_alerts, detections. |
| src/database/repository.py | Typed CRUD repository with batch insertion, automatic pruning of old records, and concurrent-safe transaction management. |
| src/dashboard/app.py | Flask application factory. Configures the WSGI app with context injection and shared FrameBuffer. |
| src/dashboard/routes.py | REST API endpoints: /api/stats, /api/behavior_counts, /api/alerts, /api/ambient, /api/settings (GET+POST). |
| src/dashboard/stream.py | MJPEG video streaming endpoint. Annotates frames with bounding boxes, behavior labels, track IDs, and confidence scores. |

# 2. Inference & Object Detection (src/inference/detector.py)

The Detector class wraps an ONNX Runtime inference session loaded from models/best.onnx. On each call to detect(frame), it performs the following pipeline:

1. Preprocessing: The input frame (BGR) is resized to 640×640, normalised to [0, 1] float32, and transposed to NCHW format (1, 3, 640, 640) for the ONNX model.

2. Inference: The session.run() call executes the YOLOv8n computation graph. On the Raspberry Pi 4B CPU, this averages ~48 ms per forward pass.

3. Postprocessing: Raw output boxes are decoded, confidence-filtered (threshold from config.yaml), and deduplicated with Non-Maximum Suppression (NMS) using an IoU threshold of 0.45.

The Detector also supports optional profiling mode (enable_profiling=True), which records per-step timing breakdowns for performance analysis.

# 3. Object Tracking (src/tracking/)

sort_tracker.py implements the SORT algorithm:

• Prediction: Each tracked bounding box is predicted forward one timestep using a linear Kalman filter (constant velocity model in [x, y, s, r] space).
• Association: Hungarian algorithm solves the assignment problem between predictions and new detections using Intersection-over-Union (IoU) as the cost metric.
• Track Lifecycle: Tracks are confirmed after min_hits=3 consecutive detections, aged out after max_age=30 missed frames.

pig_tracker.py builds on SORT to maintain a richer state per pig ID:
• Behavior: Most recent YOLO class label.
• Stationary Timer: Incremented per frame when the centroid moves < 20 pixels; reset on movement.
• Centroid History: A rolling deque of the last N centroid positions.

# 4. Health Risk Engine (src/health/risk_engine.py)

The RiskEngine runs two independent alert channels on every frame:

Channel 1 — Individual Fever Alert:
  IF (pig stationary ≥ 15 min) AND (zone_temp > ambient_temp + 2.0°C) → ALERT
  THI Adaptation: If THI > 78, the threshold extends to 30 min to prevent hot-weather false positives.

Channel 2 — Population Lethargy Alert:
  IF (stationary_pigs / total_pigs ≥ 0.60) FOR ≥ 3 consecutive seconds → ALERT

When either channel triggers, the engine calls GSMNotifier.send_alert() and logs the event to the database with a full trigger_reason string for audit purposes.

# 5. Database Layer (src/database/)

schema.py creates three tables in SQLite with WAL (Write-Ahead Logging) mode and a 5-second busy_timeout for concurrent access safety:

• ambient_readings: (id, timestamp, temp_c, humidity_pct, thi)
• pen_alerts: (id, timestamp, alert_type, trigger_reason, sms_sent, pig_id)
• detections: (id, track_id, timestamp, behavior, confidence, bbox_json, zone_temp_c)

repository.py provides batch insertion (reduces SQLite write overhead), automatic pruning of records older than a configurable retention period, and typed query functions for the dashboard API endpoints.

# 6. Flask Dashboard (src/dashboard/)

app.py creates the Flask application factory with a shared FrameBuffer — a thread-safe structure that holds the latest annotated frame for the MJPEG stream.

routes.py exposes the following REST endpoints:
• GET /              — Main dashboard page (Jinja2 rendered)
• GET /video_feed    — MJPEG live video stream (multipart/x-mixed-replace)
• GET /api/stats     — Current pig count, FPS, alert status
• GET /api/ambient   — Latest DHT22 temperature, humidity, THI
• GET /api/alerts    — Last N alerts from database
• GET+POST /settings — Live configuration editor (writes to config.yaml)

stream.py renders bounding boxes, behavior labels (#ID behavior confidence%), and thermal overlay onto each captured frame before encoding it as JPEG.