# Formulas and Weights Explanation

Mathematical Models · Decision Logic · Configuration Parameters

# 1. Temperature Humidity Index (THI)

The THI is an internationally recognised index used in animal husbandry to quantify the combined effect of temperature and humidity on heat stress in livestock. The system uses the standard NRC formula, adapted for Celsius input:

THI  =  (1.8 × T + 32)  −  [ (0.55 − 0.0055 × RH) × (1.8 × T − 26) ]

Where:

T:  Ambient air temperature in degrees Celsius (°C), read from the DHT22 sensor.

RH:  Relative Humidity as a percentage (%), read from the DHT22 sensor.

THI:  Temperature Humidity Index (dimensionless composite score).

## 1.1  THI Interpretation Thresholds

| THI Range | Stress Level | System Behaviour |
| --- | --- | --- |
| < 72 | No Heat Stress | Normal alert thresholds applied (15 min stationary limit) |
| 72 – 78 | Mild Heat Stress | Normal thresholds; awareness logging activated |
| 79 – 88 | Moderate Heat Stress | Stationary threshold extended from 15 min → 30 min |
| 89 – 98 | Severe Heat Stress | 30 min threshold; population alert sensitivity increased |
| > 98 | Extreme Heat Stress | Emergency thresholds; alert cooldown reduced |

# 2. Hybrid Risk Engine Alert Logic

The risk engine operates two independent detection channels running in parallel on every processed frame. Either channel can independently trigger an SMS alert.

## 2.1  Channel 1 — Individual Fever & Lethargy Alert

TRIGGER if:  stationary_duration(pig_i) ≥ T_stationary  AND  zone_temp(pig_i) > T_ambient + Δ_fever

Where:  T_stationary = 900 s (15 min)  ← extended to 1800 s (30 min) when THI > 78
        Δ_fever      = 2.0 °C  (configurable in config.yaml)

This channel targets individual pigs. A pig must satisfy BOTH conditions simultaneously:
(a) It has been classified as stationary (centroid displacement < 20 px) for a sustained period.
(b) Its body surface temperature (measured via AMG8833 zone) exceeds ambient temperature by more than the fever delta threshold.

## 2.2  Channel 2 — Population Lethargy Alert

TRIGGER if:  (count(stationary pigs) / count(total pigs)) ≥ 0.60  FOR ≥ 3 consecutive seconds

lethargy_ratio = stationary_count / total_count

This channel monitors herd-level behavior. If 60% or more of the detected pigs are simultaneously stationary for at least 3 consecutive seconds, it indicates a possible group illness or environmental emergency (e.g. overcrowding, gas leak, extreme heat).

# 3. YOLOv8n Confidence & NMS Parameters

The ONNX inference postprocessing applies two filtering stages to the raw model output:

| Parameter | Value | Description |
| --- | --- | --- |
| conf_threshold | 0.35 | Minimum detection confidence; boxes below this are discarded |
| iou_threshold | 0.45 | IoU threshold for Non-Maximum Suppression (NMS) |
| input_size | 640 × 640 | Fixed model input resolution (baked into ONNX graph) |
| frame_skip | 3 | Run YOLO every 4th frame; SORT tracks on all frames |
| max_age | 30 | SORT: frames a track survives without a matched detection |
| min_hits | 3 | SORT: detections needed before a track is confirmed |

# 4. Stationary Detection Formula

displacement(t) = √[ (x_t − x_{t−1})² + (y_t − y_{t−1})² ]

if displacement(t) < 20 pixels:
    stationary_duration += Δt   (increment timer)
else:
    stationary_duration = 0     (reset timer)

The centroid displacement is measured in raw pixel space between consecutive frames. The 20-pixel threshold is tuned for the typical camera placement distance from the pen; it is adjustable in config.yaml via the stationary_pixel_threshold parameter.

# 5. Model Accuracy Metrics Achieved

| Metric | Value | Target | Status |
| --- | --- | --- | --- |
| mAP50 (mean Average Precision) | 0.827 | ≥ 0.70 | ✅ Exceeded |
| mAP50-95 | ~0.60+ | > 0.50 | ✅ Met |
| Inference Time (CPU, Pi 4B) | ~48 ms | ≤ 150 ms | ✅ Met |
| ONNX Export Speedup | +57.2% vs PyTorch | > 30% | ✅ Exceeded |
| ONNX Model Size | 11.7 MB | < 20 MB | ✅ Met |
| Effective Processing FPS | 25–30 FPS (with skip) | ≥ 5 FPS | ✅ Far Exceeded |