# Schematic Diagram: Physical Component Connections

Hardware Wiring · Signal Paths · Power Distribution · Pin Mapping

# 1. Overview of Physical Connections

The Offline AI Swine Health Monitoring System integrates multiple hardware components connected to a central Raspberry Pi 4B. Power is distributed from a 3S lithium battery pack through a BMS and a buck converter to all subsystems. The diagram below describes the exact physical signal and power connections between each component.

# 2. Power Distribution Network

All system power originates from a 3S3P 18650 lithium battery pack (nominal 11.1 V, ~21 Ah). A 3S 20A Battery Management System (BMS) protects the cells against over-charge, over-discharge, and short-circuit. A CN3791 MPPT solar charger continuously replenishes the pack from a 12 V solar panel. An XL4016 buck converter steps the 11.1–12.6 V battery voltage down to a stable 5.1 V / 5 A supply for the Raspberry Pi and all 5 V peripherals. One rocker switch placed on the positive battery lead acts as the master power cut-off.

| From | To | Voltage | Notes |
| --- | --- | --- | --- |
| 12 V Solar Panel | CN3791 MPPT Solar Charger — IN+/IN− | 12 V DC | MPPT tracks peak power point |
| CN3791 MPPT Charger — BAT+/BAT− | 3S3P 18650 Battery Pack | 12.6 V (full) | Constant-current / constant-voltage charging |
| 3S3P 18650 Pack | 3S 20A BMS — B+/B− | 11.1–12.6 V | BMS balances and protects all cells |
| 3S 20A BMS — P+/P− | Rocker Switch → XL4016 Buck IN+/IN− | 11.1–12.6 V | Master power on/off switch on positive rail |
| XL4016 Buck OUT+/OUT− | Raspberry Pi 4B USB-C 5 V IN | 5.1 V / 5 A | Adjust XL4016 trim pot to 5.1 V before connecting Pi |
| Raspberry Pi 5 V Header (Pin 2/4) | AMG8833, DHT22 VCC | 5 V | Pi 5 V rail powers low-current 5 V sensors |
| Raspberry Pi 3.3 V Header (Pin 1) | AMG8833 VCC (alt), DHT22 VCC (alt) | 3.3 V | Use 3.3 V option only if sensor is 3.3 V rated |
| XL4016 Buck OUT or Battery Pack | GSM900A VCC | 3.7–4.2 V (regulated) | GSM draws up to 2A burst; requires dedicated supply |

# 3. Raspberry Pi 4B GPIO & Interface Connections

The Raspberry Pi 4B serves as the central compute hub. All sensors and communication modules connect directly to its GPIO header (40-pin) or USB ports. The physical pin numbers below refer to the board pin numbering (not BCM GPIO numbers).

| Component | Signal Pin | Pi Physical Pin | Pi BCM GPIO | Protocol |
| --- | --- | --- | --- | --- |
| AMG8833 Thermal Sensor | SDA | Pin 3 | GPIO 2 (SDA1) | I2C |
| AMG8833 Thermal Sensor | SCL | Pin 5 | GPIO 3 (SCL1) | I2C |
| AMG8833 Thermal Sensor | VCC | Pin 1 (3.3V) | — | Power |
| AMG8833 Thermal Sensor | GND | Pin 6 | — | Ground |
| AMG8833 Thermal Sensor | INT (optional) | Pin 11 | GPIO 17 | Interrupt (unused) |
| DHT22 Temp/Humidity | DATA | Pin 7 | GPIO 4 | Single-Wire (10kΩ pull-up to VCC) |
| DHT22 Temp/Humidity | VCC | Pin 2 (5V) | — | Power |
| DHT22 Temp/Humidity | GND | Pin 9 | — | Ground |
| GSM900A Module | TX (Module TX) | Pin 10 | GPIO 15 (RXD) | UART RX |
| GSM900A Module | RX (Module RX) | Pin 8 | GPIO 14 (TXD) | UART TX |
| GSM900A Module | GND | Pin 14 | — | Shared Ground |
| GSM900A Module | VCC | Dedicated PSU | — | 3.7–4.2 V from voltage divider/regulator |
| Logitech 1080p Webcam | USB | USB 3.0 Port | — | USB UVC (/dev/video0) |

I2C SETUP: Run "sudo raspi-config" → Interface Options → I2C → Enable. Verify AMG8833 is detected: "sudo i2cdetect -y 1" (expect address 0x69 or 0x68).
UART SETUP: Run "sudo raspi-config" → Interface Options → Serial Port → Disable login shell; Enable hardware serial. Add "enable_uart=1" to /boot/config.txt.

# 4. Solar Charging & Battery Management Connections

The CN3791-based MPPT solar charger manages energy flow from the solar panel to the battery pack. The BMS ensures safe operation of all three cell groups in the 3S3P configuration. Correct wiring of the BMS is critical — reversing the charge/discharge ports will damage the BMS permanently.

| Connection From | Connection To | Notes |
| --- | --- | --- |
| Solar Panel Positive | CN3791 IN+ terminal | 12 V (open-circuit ~18 V) |
| Solar Panel Negative | CN3791 IN− terminal | Common ground |
| CN3791 BAT+ | 3S BMS B+ (Battery Positive) | Charging current path |
| CN3791 BAT− | 3S BMS B− (Battery Negative) | Common ground |
| 3S BMS P+ (Load Positive) | Rocker Switch IN | Protected discharge output |
| Rocker Switch OUT | XL4016 Buck Converter IN+ | Switched 12 V bus |
| 3S BMS P− (Load Negative) | XL4016 Buck Converter IN− | Common ground |
| XL4016 OUT+ | Raspberry Pi USB-C 5V IN (via cable) | Regulated 5.1 V |
| XL4016 OUT− | Raspberry Pi GND / Common Ground | Common ground reference |

# 5. Signal Summary

| Signal Path | Interface | Description |
| --- | --- | --- |
| USB Camera → Pi | USB 3.0 | MJPEG / YUV2 video stream at 1920×1080 (downscaled to 640×480 in software) |
| AMG8833 → Pi | I2C (400 kHz) | 64-pixel (8×8) thermal array, polled at ~10 Hz by main loop |
| DHT22 → Pi | Single-wire GPIO | Temperature (°C) + Humidity (%) every 2 seconds, background thread |
| Pi → GSM900A | UART TX | AT commands at 9600 baud (SMS sending, signal check) |
| GSM900A → Pi | UART RX | AT command responses and incoming SMS notifications |
| Pi → Smartphone/PC | Wi-Fi / Ethernet (TCP) | HTTP Flask dashboard + MJPEG stream on port 5000 |